## Trap Equipment

- Two in-line 8-foot diameter rotary screw traps

## Current Trap Location

- Lower Feather River (RM 17) near Star Bend Park and Boat Ramp near Olivehurst, California

## Monitoring Schedule

- October through June

- Continuously trapping except under high flows and unsafe working conditions

- Serviced daily within a 24-hour period, more if needed due to high debris amounts or fish catch

- Note: data within the current year’s monitoring season are considered provisional

## Daily Environmental Data

- Flow (cubic feet per second), recorded from the CDEC Gauging station: Feather River at Boyd’s Landing Above Star Bend (FSB)

- Water temperature (Fahrenheit/Celsius)

- River Depth (inches/feet)

- Turbidity (nephelometric turbidity units)

- Dissolved Oxygen (milligrams per liter)

- Conductivity (microSiemens per centimeter)

- Velocity (meters per second)

## Fish Measurements

Fork length measurements to the nearest mm are taken for:

- Chinook salmon: 50 randomly selected per category: marked (with an adipose fin clip) spring-, fall-, winter- or late fall-runs; unmarked spring-, fall-, winter- or late fall-runs. Run designated by Length at Date criteria (Fisher 1992).

- Steelhead: 50 randomly selected per marked or unmarked

- Non-salmonid fish: first 10 (total length measurement to nearest mm)

## Fish Counts

- All fish not measured are plus counted by hand.

## Trap Efficiency Trials

- Fall-run Chinook salmon captured in the RSTs or juvenile Chinook salmon produced by the Feather River Fish Hatchery are the source of released fish.

- Frequency of once per week from December to May across variable flow and environmental conditions as catch totals and/or hatchery availability allows

- A minimum of three consecutive days of fishing after the release date is required for a release to be included in analyses.

- Mortality between the release point and the trap is assumed to be negligible.

- Release site approximately 1 mile upstream of the trapping site.

## Trap Efficiency Marks

- Fish are marked with Bismarck Brown Y (BBY) whole body stain.

- Fish are alternatively marked with a Visible Implant Elastomer (VIE) Tag.

## References

- Fisher, F. W. 1992. Chinook salmon, Oncorhynchus tshawytscha, growth and occurrence in the Sacramento-San Joaquin River system. Draft report dated June 1992. Sacramento, CA: Inland Fisheries Division, California Department Fish and Game. 42 p.

Note: data within the current year’s monitoring season are considered provisional.